Grandstream Networks， Inc.

GWN 700x Dynamic Routing

User Guide v1.0

Table of Contents

OSPF	3

Dynamic Routing Learning	3

Configuration	3

Show OSPF Route learning	5

Neighbor Info	5

Test Network Connectivity	6

Import Direct Routing	6

Configuration	7

Show OSPF Route learning	9

Test Network Connectivity	10

RIP	11

Dynamic Routing Learning	11

Configuration	11

Neighbour Info	12

Test Network Connectivity	13

Import Direct Routing	13

Configuration	14

Test Network Connectivity (PC1 ping PC2)	15

BGP	16

Dynamic Routing Learning	16

Configuration	16

Peer Info	17

Test Network Connectivity（PC1 ping PC2）	18

Import Direct Routing	18

Configuration	18

Show BGP Route learning	20

Test Network Connectivity (PC1 ping PC2)	21

OSPF

Dynamic Routing Learning

The example of networking scenario:

Configuration

Global Settings

- Navigate to Routing→OSPF→Global Settings

- Enable "OSPF" and fill in the Router ID

- Click Save

Interface Settings

On this tab, users can add interfaces (e.g VLANs) or enable/disable them, and perform related configurations based on the interface. It mainly includes area ID, Hello interval, Dead interval, overhead

value, etc.

- Navigate to Routing->OSPF->Interface Settings

- Users can add interfaces (e.g VLANs) or enable/disable them, and perform related configurations based on the interface. It mainly includes area ID, Hello interval, Dead interval, overhead value, etc.

- Click "Save" to apply

Note:

If global OSPF is not enabled, the configuration will not take effect.

At the same time, OSPF and interface information should also be configured on the two switches on the LAC side

Note: Both VLAN30 on Switch1 and VLAN4 on Switch2 need to configure the interface and enable OSPF

Area Settings

- Navigate to Routing->OSPF->Area Settings

- The area ID configured in the interface Settings will be displayed synchronously in this section, and the area type can be configured.

Show OSPF Route learning

- Navigate to Routing->Routing Table

- After the configuration is complete, check the routing table. It can be seen that the VLAN5 and VLAN30 routes under the two switches have been learned, and the next hop is the IP address of the corresponding switch, respectively

- At the same time, you can go to the routing table of the switch to view it, and the switch can also learn VLAN 60 of the router

Neighbor Info

All neighbor information currently learned can be viewed in the neighbor table

Test Network Connectivity

According to the above networking scenario, PC1 and PC2 and PC3 can ping each other, and the results are as follows:

Import Direct Routing

The example of networking scenario

Configuration

GWN7003A is configured as follows:

- Enable OSPF and configure the Router ID.

- Add interfaces: Add Default (vlan) and VLAN60 interfaces, and OSPF is enabled.

GWN7003B is configured as follows:

- Enable OSPF and configure RouterID (make sure it is unique)

- Enable direct routing

- Add interface: Add WAN interface and OSPF is enabled;

Note: Because direct routing is enabled, VLAN40 does not need to configure interfaces

（See the previous scenario for detailed configuration steps）

If you want to access devices under VLAN40,GWN7003B needs to configure firewall rules：

- Firewall->Traffic Rules->Forwarding Rules Add a forwarding rule

Show OSPF Route learning

Navigate to Routing Table, OSPF routing learning results can be viewed

Test Network Connectivity

RIP

Dynamic Routing Learning

The example of networking scenario

Configuration

Global Settings

Users can configure the RIPv1 or RIPv2 global parameters like enabling RIP, RIP Distance, and Timers (Update Timer, Invalid Timer and Garbage Collection Timer) as well as External Route Import (Direct, Static, OSPF and BGP).

To configure RIP, please navigate to Routing → RIP→Global Settings:

Interface Settings

The Interface Settings tab allows the user to Add/Delete/Edit interfaces (e.g VLANs), enable the RIP on the interface and perform related configurations.

Note:

If global RIP is not enabled, the configuration will not take effect.

Route Advertisements

RIP route advertisement involves broadcasting routing information to neighboring routers, providing details about network destinations and associated metrics, facilitating dynamic route updates in a network. RIP advertises routes periodically to maintain an updated routing table.

The advertised routes will be shown on this tab, the user has the option to add a route advertisement by clicking on “Add” button.

Neighbour Info

This page displays the neighbor information table under the current RIP process.

Test Network Connectivity

Import Direct Routing

The example of networking scenario

Configuration

GWN7003 is configured as follows:

- Enable RIP

- Enable External Route Import and Protocol Type Select Direct

- Add interface: Add WAN interface

- Add Route Advertisement

If PC2 want to access devices under PC1,GWN7003 needs to configure firewall rules：

- Firewall->Traffic Rules->Forwarding Rules. Add a forwarding rule

Test Network Connectivity (PC1 ping PC2)

BGP

Dynamic Routing Learning

The example of networking scenario

Configuration

Global Settings

On the Global Settings tab, the users can enable BGP protocol and the set the AS (Autonomous System) and the Router ID, as well as importing external routes: Direct, Static, OSPF and RIP.

To configure BGP, please navigate to Routing→BGP→Global Settings:

Note:

If you want to configure IBGP neighbors, then the AS numbers of two 7003 are the same. If you want to configure EBGP neighbors, then the AS numbers of two 7003 are different

Peer

When using the BGP protocol, a peer relationship must be established to exchange BGP messages between devices. The user can add a peer by clicking the “ Add ” button or modify the previously added peers and perform related configurations.

Note:

- If global BGP is not enabled, the configuration will not take effect.

- ‘Remote AS’, please enter the AS of the remote device

Route Advertisement

This page allows the user to set the subnet segment address to be advertised using the BGP protocol. To add more, click the “+” icon.

Peer Info

On this page, the users can find the peer status information displayed with BGP Version Info, Peer Address, Peer AS, Status and Uptime.

Test Network Connectivity（PC1 ping PC2）

note：

If you want to access devices under VLAN,GWN7003A and GWN7003B needs to configure firewall rules：

- Firewall->Traffic Rules->Forwarding Rules. Add a forwarding rule from wan –to-lan

Import Direct Routing

The example of networking scenario

Configuration

GWN7003 is configured as follows:

- Enable BGP and configure AS and RouterID (make sure it is unique)

- Enable direct routing

- Add peer: configure remote AS and and remote address, BGP status is enabled

Note:

If you want to configure IBGP neighbors, then the AS numbers of two 7003 are the same. If you want to

configure EBGP neighbors, then the AS numbers of two 7003 are different

Show BGP Route learning

Navigate to Routing Table, BGP routing learning results can be viewed

Test Network Connectivity (PC1 ping PC2)